Contour-based Particle Detection and Inter-particle Distance Measurement

In this study, a classical computer vision pipeline was developed to detect spherical micro-particles in microscopy videos and quantify inter-particle distances over time. The pipeline integrates colour-space filtering, adaptive thresholding, hierarchical contour extraction (Suzuki & Abe, 1985), and pairwise shortest-distance estimation. The transformation from input frame I to particle mask M is expressed as:

M = Φhier(Ψthr(G(R(I))), amin, amax)

Where R(·) is the HSV red-shadow attenuation operator, G(·) the grayscale projection with optional CLAHE (Zuiderveld, 1994), Ψthr(·) the adaptive Gaussian thresholding operator (Bradley & Roth, 2007), and Φhier(·) the hierarchical contour filter constrained by [amin, amax].

Architecture of the Proposed Pipeline

The pipeline comprises four sequential modules. The red-shadow attenuation module converts each frame from BGR to HSV, builds a red-region mask by combining the hue intervals [0°, 15°] and [160°, 180°] via bitwise OR, refines it through morphological closing and opening with a 5×5 kernel, and attenuates the saturation channel within the mask by a factor of three. The binarisation module converts the desaturated frame to grayscale, optionally enhances local contrast via CLAHE (clip limit 5.0, 8×8 grid), and applies adaptive Gaussian thresholding with block size 51 and constant C = 25. The binary image is inverted and refined through a dilation–erosion cycle with a 3×3 kernel.

The contour extraction module invokes cv2.findContours with the RETR_TREE retrieval mode and retains contours satisfying (i) amin < area < amax (defaults 20 px² and 1000 px²) and (ii) a non-negative parent index, isolating enclosed inner holes corresponding to particle cross-sections. The distance estimation module computes, for every unordered pair (Ci, Cj), the shortest Euclidean boundary distance via adaptive sub-sampling with ns = max(10, min(50, |C|/10)):

d(Ci, Cj) = min ‖p − q‖₂, p ∈ S(Ci), q ∈ S(Cj)

Pixel distances are converted to micrometres through a calibrated factor of 6.2 px/μm. Five per-frame statistics jointly characterise the spatial distribution and aggregation tendency of the particle ensemble: the minimum, maximum, mean, and median pairwise distance, together with a close-pair ratio defined as the proportion of pairs satisfying d < 3 μm relative to the sub-population of pairs with d ≤ dmedian. Restricting the denominator to this near-neighbour sub-population removes the dilution caused by long-range pairs that carry no aggregation information, yielding a statistic more sensitive to local clustering behaviour.

Methods

Microscopy videos were decoded frame-by-frame via the OpenCV VideoCapture interface (Bradski, 2000) and processed independently. Hyper-parameters—block size, constant offset C, area interval, and CLAHE clip limit—were tuned by an interactive grid search on representative frames, selecting the configuration with the lowest false-positive rate. Three CSV records were generated per video, and scatter plots of each statistic against the frame index were rendered via Matplotlib (Hunter, 2007). The combined strategy substantially improved detection robustness under heterogeneous illumination.

References

Bradley, D., & Roth, G. (2007). Adaptive thresholding using the integral image. Journal of Graphics Tools, 12(2), 13–21.

Bradski, G. (2000). The OpenCV library. Dr. Dobb’s Journal of Software Tools, 25(11), 120–125.

Hunter, J. D. (2007). Matplotlib: A 2D graphics environment. Computing in Science & Engineering, 9(3), 90–95.

Suzuki, S., & Abe, K. (1985). Topological structural analysis of digitized binary images by border following. Computer Vision, Graphics, and Image Processing, 30(1), 32–46.

Zuiderveld, K. (1994). Contrast limited adaptive histogram equalization. In Graphics Gems IV (pp. 474–485). Academic Press.